Aluminium Frame Neo Advance® Flat Fixed Rooflight units __ degree flat roofs

- Manufacturer: The Rooflight Co.

  - Contact Details

    - Address: Unit T1 Bourton Industrial Park

Bourton-on-the-Water

Cheltenham

GL54 2HQ

    - Telephone: +44 (0) 1993 833155

    - Web: www.therooflightco.com

    - Email: hello@therooflightco.com

  - Product Reference: Neo Advance Flat Fixed (NFF___)

- Product Performance

  - Whole Window U-Value: 1.72W/m2K

- Frame

  - Material: Patented Aluminium Frame (as per GB2586514 and GB2586515)

  - Finish

    - Corrosion Protection: EN 60068-2-11 to classification 1440hrs (60 days).

    - Coating: Polyester Powder Coating

    - Colour: RAL 7022 Smooth Semi-Gloss Umbra Grey

- Kerb Installation: As per manufacturer’s recommendations

- Glazing or Infill

  - Standards/Type: BSI Kitemarked (KM93754) Insulated glazing units to EN1279

  - Construction: 6HT (20 Argon 90) 33.1

  - Colour: Clear

  - Acoustic Performance of Glazing: 37 (-2; -6) dB to EN12758

- Operation

  - Operation Type: Fixed Closed

- Accessories: N/A

- Glazing Bars: N/A

- Third Party Certification: N/A