Steel frame Neo Rooflight units ____ degree pitched roofs

- Manufacturer: The Rooflight Co.

  - Contact Details

    - Address: Unit T1 Bourton Industrial Park

Bourton-on-the-Water

Cheltenham

GL54 2HQ

    - Telephone: +44 (0) 1993 833155

    - Web: www.therooflightco.com

    - Email: hello@therooflightco.com

  - Product Reference: Neo Rooflight (Neo ___)

- Product Performance

  - Whole Window U-Value: 1.4W/m2K

- Frame

  - Material: Patented Steel Frame (as per GB251419)

  - Finish

    - Corrosion Protection: EN ISO 12944 to classification C5-M Long.

    - Coating: Polyester Powder Coating

    - Colour: RAL 7022 Smooth Semi-Gloss Umbra Grey

- Glazing or Infill

  - Standards/Type: BSI Kitemarked (KM93754) Insulated glazing units to EN1279-3

  - Construction: 4HT (16 Argon 90) 4HT

  - Colour: Clear

  - Acoustic Performance of Glazing: 31 (-1; -4) dB to EN12758

- Operation

  - Operation Type: Fixed Closed/ Brushed Stainless Steel Manual Handle/ Motorised Opening with Concealed Jamb Motors to stroke length of 150/300/450mm/ Motorised Opening with Concealed Cill Motor to stroke length of 150/300/450mm [Delete as Appropriate]

- Accessories: Concealed Gas Struts/ N/A [Delete as appropriate. Select N/A if fixed closed opening option is selected.]

- Glazing Bars: N/A

- Third Party Certification: Conformity Marking in accordance with EN 14351-1. BSI Kitemark on glazing KM93754.